NETWORK ADMINISTRATION

LAB CLASS JOURNAL

TNE10005

RUBIE STANNARD 103982732

Table of Contents

- Week 1	3

1.1. Key Concepts	3

1.2. Key Configurations and Commands	3

1.3. Further Study	3

- Week 2	5

2.1. Key Concepts	5

2.2. Key Configurations and Commands	5

2.3. Further Study	5

- Week 3	7

3.1. Key Concepts	7

3.2. Key Configurations and Commands	7

3.3. Further Study	7

- Week 4	9

4.1. Key Concepts	9

4.2. Key Configurations and Commands	9

4.3. Further Study	10

- Week 5	11

5.1. Key Concepts	11

5.2. Key Configurations and Commands	11

5.3. Further Study	12

- Week 6	13

6.1. Key Concepts	13

6.2. Key Configurations and Commands	13

6.3. Further Study	14

- Week 7	15

7.1. Key Concepts	15

7.2. Key Configurations and Commands	15

7.3. Further Study	16

- Week 8	17

8.1. Key Concepts	17

8.2. Key Configurations and Commands	17

8.3. Further Study	19

- Week 9	20

9.1. Key Concepts	20

9.2. Key Configurations and Commands	20

9.3. Further Study	21

- Week 10	22

10.1. Key Concepts	22

10.2. Key Configurations and Commands	22

10.3. Further Study	22

- Week 11	23

11.1. Key Concepts	23

11.2. Key Configurations and Commands	23

11.3. Further Study	23

- Week 12	24

12.1. Key Concepts	24

12.2. Key Configurations and Commands	24

12.3. Further Study	24

References	26

- Week 1

1.1.    Key Concepts

Week one was used as an introduction to Network Administration. It introduced us to virtual machines, “a computer resource that uses software instead of a physical computer to run programs and deploy apps,”[1] and the different applications that allow people to run them. Effective for organisations and businesses, virtual machines save space, money, and time, and may act as a sort of safeguard as they allow users to test applications, programs, and links on the virtual host before using or downloading them on the physical host.

Some applications that people can use to run virtual machines, which are the applications we will be using throughout Network Administration, are Hyper-V Manager, “an administrative tool used for creating, modifying, and removing virtual machines and hosts running in a Hyper-V virtual environment,”[2] and Azure Labs, a service that provides “access to preconfigured virtual machines.”[3]  Both of these are Microsoft services.

1.2.    Key Configurations and Commands

This week’s lab exercise was about an introduction to the Network Administration labs. During the lab exercise, the following configurations and commands were learnt:

- Pa55w.rd: Used as the standard password for all user accounts.

- Command prompt: Right click the Windows key  Run  type cmd.

- ipconfig: Used to look at a VM’s ethernet adapter IP configuration when typed in the command prompt.

- Configure an IP address and subnet mask: Server Manager  Local Server  click the link next to Ethernet below the Properties section  Network Connections  Ethernet  Properties  Internet Protocol Version 4 (TCP/IPv4)  Properties  Enter the specified IP address and configuration details  OK twice.

1.3.    Further Study

Binary seems like it’ll be a big part of Network Administration, and this was something the class teacher touched on a bit. Converting binary to an IP address and calculating what number a binary piece represents and vice versa is something that I’m uncertain about, and something that I should study further. The tables below are some things the teacher wrote on the board that I thought I should copy down.

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | Byte |
| 128 | + | 32 | + | 8 | + | 2 + | 1 | = 171 |

| Bit | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- |
|  | 1 | 0 | 1 | 0 |
|  | 8 | + | 2 |  |
|  | = | 10 | = | A |

| Decimal | n0nnn1nnn2nnn3nnn4nnn5nnn6nnn7nnn8nnn9nnn10nnn11nnn12nnn13nnn14nnn15 |
| --- | --- |
| Hex | n0nnn1nnn2nnn3nnn4nnn5nnn6nnn7nnn8nnn9nnnAnnnnBnnnnCnnnnDnnnnEnnnnF |

Figure 1: Converting binary table.

Although I’ll have to study multiple topics covered in the lecture 01 revision quiz, in my opinion, binary seems like it’ll be the toughest thing to understand, therefore needing a bit more focus than the other topics.

- Week 2

2.1.    Key Concepts

Week two was still about being introduced and getting familiar with Network Administration. This was done through observing the different settings and configurations for each virtual machine. Looking at settings allows us to:

- Understand how devices communicate.

- Make sure the settings are correct.

- Ensure everything is on.

- Observe the different components of networking.

Although this lab was simple, examining settings is an important part of networking.

2.2.    Key Configurations and Commands

This week’s lab exercise was about getting familiar with the Network Administration labs. During the lab exercise, the following configurations and commands were learnt:

- Revert: Used to bring a VM back to its preinstalled connections, settings, and roles.

- ping <IPv4 address of VM>: Used to verify communication between VM’s. A successful ping will return Reply from <address>: bytes=32 time=1ms TTL=128.

- ipconfig /all: Used to show all the Windows IP Configuration and Ethernet adapter information.

- netsh interface ip set address Ethernet dhcp: Used in Windows PowerShell (Admin) to automatically obtain IP settings from a DHCP server.

2.3.    Further Study

The lecture 02 revision quiz was mainly about device communication and IP addresses. It touched on layers as well, but the communication and IP address related questions are what I need to focus on. The first image is asking what two devices can communicate with each other. The answer was laptop 1 to PC3.

Figure 2: Device communication.

Based on their IP addresses and subnet masks, Laptop 1 (10.10.1.20 255.255.0.0) and PC3 (10.10.1.102 255.255.255.0) are both in the same subnet (10.10.1.0/24), meaning they share the same network segment and can directly communicate with each other. Devices within the same subnet can communicate using their MAC addresses. Since Laptop 1 and PC3 are within the same subnet, they can use their MAC addresses to communicate. Therefore, Laptop 1 can send data to PC3 by using PC3's MAC address as the destination MAC address in the Ethernet frame.

The second image is about PC’s sending packets to each other. The question asked what address is placed in the frame’s destination address if PC10 sends a packet to PC21. The answer was Router Interface 0’s MAC address 0A-11-22-33-4E-11.

Figure 3: Frame destination addresses.

When PC10 wants to send a packet to PC21, the communication involves different devices in the network. In order to send data between devices, they use MAC addresses. The frame's destination address should be the MAC address of the next-hop device which will help route the packet towards its final destination. Router Interface 0 (MAC address: 0A-11-22-33-4E-11) is the device that connects PC10's subnet to PC21's subnet. Therefore, PC10 will use Router Interface 0's MAC address as the destination address in the frame, to ensure that the packet gets to the router which will then forward it towards PC21.

The last part I would like to focus on is finding the network address of an IP address. One of the questions was asking what the network address of 136.184.203.12 255.255.192.0 is. To find network address of an IP address:

- If we have the IP address 136.184.203.12/18, the subnet mask (/18) would be 255.255.192.0. In order to find the network address, keep the first two octets of 136.184.203.12, and change the last  two octets to the last two octets of the subnet mask, e.g., keep 136.184 and change 203.12 to 192.0. The network address is 136.184.192.0.

As mentioned above, I will need to study the other question topics such as layers, but IP addresses and device communication is what I’ll chose to study more.

- Week 3

3.1.    Key Concepts

Week three introduced us to IPv4 (Internet Protocol version 4) addressing that consists of a network and host portion and uses “a 32-bit string of numbers to uniquely identify network interfaces in a device” which are needed for devices “to be able to communicate with each other.”[4]

Week three also mentioned default gateways and MAC addresses, aspects that are needed for a network to function and communicate. Default gateways “allow devices on separate networks to communicate with one another”[5] through computer nodes, and MAC addresses allow “communications within network segments” by assigning a 12-character alphanumeric “unique identifier to network interface controllers for use as network addresses.”[6]

3.2.    Key Configurations and Commands

This week’s lab exercise was about an introduction to IPv4 addressing. During the lab exercise, the following configurations and commands were learnt:

- Adapter configuration: File Explorer  Network  Properties  Ethernet  Properties  Internet Protocol Version 4 (TCP/IPv4).

- arp -a: Used to display the ARP table.

- arp -d: Used to delete an entry from the ARP table.

3.3.    Further Study

This week was focused on subnetting a lot and the different parts, such as broadcast addresses, subnet ID’s and subnet masks, and I have written an example for each of these. The first is how to find the broadcast address. The image below demonstrates how to find the broadcast address.

Figure 4: How to find the broadcast address.

The broadcast address of the IP address 10.20.30.40 with a subnet mask of /16 (255.255.0.0) is 10.20.255.255. To find the broadcast address:

- Convert the subnet mask to binary. The subnet mask /16 (255.255.0.0) is equivalent to 16 bits set to 1 followed by 16 bits set to 0 in binary: 11111111.11111111.00000000.00000000

- Calculate the subnet prefix. The IP address 10.20.30.40 in binary is: 00001010.00010100.00011110.00101000

- Applying the subnet mask, the subnet prefix becomes: 00001010.00010100.00000000.00000000

- Calculate the host portion. The host portion of the IP address (last 16 bits) is: 00011110.00101000

- Calculate the broadcast address. To find the broadcast address, you need to set all the host portion bits to 1 within the subnet prefix: 00001010.00010100.11111111.11111111

- Convert the binary broadcast address back to decimal: 00001010.00010100.11111111.11111111 translates to 10.20.255.255

Another thing we learnt was how to find the subnet ID. If we have the IP address 10.20.30.40/16, the subnet mask (/16) would be 255.255.0.0. In order to find the subnet ID, keep the first two octets of 10.20.30.40, and change the last two octets to the last two octets of the subnet mask, e.g., keep 10.20 and change 30.40 to 0.0. The subnet ID is 10.20.0.0.

Next was how to find the subnet mask. A Server cannot communicate with other subnets in the network. It has the IP address 136.186.28.245 and the subnet mask of 255.255.255.0. You suspect that the PC has been configured incorrectly. You know that the subnet ID is 136.186.28.128 and that the broadcast address for the subnet is 136.186.28.255. What should be the correct subnet mask? To find the subnet mask of an IP address:

- The host of 136.182.28.245 when ANDed with the subnet mask 255.255.255.0 results in the subnet ID 136.182.28.0: 136.182.28.245 AND 255.255.255.0 = 136.182.28.0

- The subnet ID (first address) is meant to be 136.186.28.128, which means the subnet mask must be configured wrong. The first three octets match so the issue is in the last octet.

- We know the broadcast (last) address is 136.186.28.255, so we can figure out what the subnet gap for the fourth octet is. We have to add 1 to the broadcast ID to get the next subnet's ID to calculate the gap: (255 + 1) - 128 = 128.

- 128 in binary is 1000 0000, so we know we only need one extra network bit for the subnet mask in the fourth octet. This works out in CIDR notation as: /24 + 1 = /25 or 255.255.255.128

- If we try the ANDing again with the new subnet mask, you'll see we now get the correct subnet ID of 136.182.28.245 AND 255.255.255.128 = 136.182.28.128

The lecture 03 revision quiz had some IP address and subnet questions, as well as questions about socket components, TCP protocols and matching port numbers, and TCP/IP protocols and layers. If topics such as these are simple like they were on the quiz, I shouldn’t have to study them that much.

- Week 4

4.1.    Key Concepts

Week four introduced us to the Dynamic Host Configuration protocol and the multiple parts, such as scopes, exclusions, and option, that are part of DHCP servers. DHCP is a “protocol that automatically provides an IP host with its IP address and other related configuration information.”[7]

Some parts that make up the DHCP protocol are scopes which are ranges of “IP addresses that a DHCP servers can draw on to fulfill IP address requests from DHCP clients,”[8] reservations, “specific IP addresses that are tied to certain devices through MAC addresses,”[8] and exclusions, which are “addresses taken from DHCP scopes that DHCP servers are not allowed to hand out.”[9]

All of these are important for businesses because as they provide reliable configuration information and allow you to control aspects of your virtual network.

4.2.    Key Configurations and Commands

This week’s lab exercise was about DHCP in a Windows Server 2022 domain. During the lab exercise, the following configurations and commands were learnt:

- Set-NetIPInterface –InterfaceAlias Ethernet –dhcp enabled: Used to set a static IP address (NIC) to DHCP.

- Install DHCP: On sWin22SVR1 go to Server Manager  Manage  Add Roles and Features  next until you’re at Select Server Roles  tick the check box next to DHCP  Add Features  Next three times  confirm installation selections  verify you’re adding the DHCP server tools  install.

- Configure DHCP server roles: On sWin22SVR1 go to Server Manager  click the yellow triangle alert  Complete DHCP configuration  DHCP Post-install configuration wizard  Next  Authorization  accept default section  Commit  Summary page  Close.

- Create a scope: On sWin22SVR1 go to Server Manager  Tools  DHCP  click the server name  right-click IPv4  New Scope  Scope wizard  Next  enter the specified name and description  Next  enter the specified values on the IP Address Range page  Next  configure the specified exclusions on the Add Exclusions and Delay page  Next  Configure DHCP Options  No  Next  Finish  DHCP Management console  right click the Scope icon  Activate from the context menu  on sWin22DC1 go to Server Manager  Tools  DHCP console  right-click sWin22dc1.swin.local  All Tasks  Stop  on sWin10CL101 go to PowerShell  ipconfig /release  ipconfig /renew.

- ipconfig /renew: Used to test if a DHCP server is working.

- ipconfig /release: Used to “send a DHCPRELEASE message to the DHCP server to release the current DHCP configuration and discard the IP address configurations.”[10]

- Configure an exclusion: On sWin22SVR1 go to DHCP management console  expand the scope  right-click the Address Pool container  New Exclusion Range  enter the specified start and end IP addresses  Add  on sWin10CL101 go to PowerShell  ipconfig /release  ipconfig /renew.

- Configure a reservation: On sWin22SVR1 go to DHCP Management  expand IPv4 and Scope  right-click Reservations  New Reservation  enter sWin10CL101, specified IP address and sWin10CL101’s MAC address  on sWin10CL101 go to PowerShell  ipconfig /release  ipconfig /renew.

- Configure DHCP options: On sWin22SVR1 go to DHCP Management console  expand the IPv4 scope  right-click Server Options  context menu  Configure Options  configure the specified options in Server Options dialog box  on sWin10CL101 go to PowerShell  ipconfig /release  ipconfig /renew.

- Configure scope options: On sWin22SVR1 expand the scope  right-click Scope Options  context menu  Configure Options  configure the specified information  Add  on sWin10CL101 go to PowerShell  ipconfig /release  ipconfig /renew.

- Configure reservation options: On sWin22SVR1 expand Scope  expand Reservations  click the specified IP address  right-click to bring up the context menu  Configure Options  configure the specified configurations  OK.

- Configure option precedence: On sWin22SVR1 go to DHCP management console  configure the specified server option, scope option, and reservation DNS domain names  on sWin10CL101 go to PowerShell  ipconfig /all.

4.3.    Further Study

The lecture 04 revision quiz had a lot of DHCP and IP address related questions. Some DHCP questions were about identifying errors after installing DHCP servers, DHCP packet order, DHCP options, etc, and I was alright with most of these questions. Some DHCP questions such as what the DHCP configurations would be after an ipconfig /renew and DHCP renewal are where I went wrong.

The IP address related questions were about borrowing bits to create subnets for a number of branches, having bits and needing to know how many hosts each subnet will support, supernetting, and more are the questions I had the most trouble with. Like the previous weeks, most of my focus will need to go towards IP address and subnet related study.

The teacher also drew another table on the board. I assume it’s about converting binary to find the CIDR notation. Here is the table:

| 3rd Octet | 4th Octet |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| /17 | 128 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| /18 | 192 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| /19 | 224 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| /20 | 240 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| /21 | 248 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| /22 | 252 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

|  | Subnet Size |  | CIDR |
| --- | --- | --- | --- |
| 256 – 128 = | 128 | – 2 = 126 | /25 |
| 256 – 192 = | 64 | – 2 = 32 | /26 |
| 256 – 224 = | 32 | – 2 = 30 | /27 |
| 256 – 240 = | 16 | – 2 = 14 | /28 |
| 256 – 248 = | 8 | – 2 = 6 | /29 |
| 256 – 252 = | 4 | – 2 = 2 | /30 |

Figure 5: Binary to CIDR table.

- Week 5

5.1.    Key Concepts

Week five introduced us to Domain Name Systems (DNS) and the network resources that are needed for DNS to work. These resources included primary forward lookup zones, DNS records, zone transfers, Internet Information Services (IIS), and CNAME records.

We first looked at DNS zone hosts, which “records resolve names to IP addresses. It’s the authoritative source for information about the domain name managed by that zone.” Relating to zones are primary forward lookup zones that “allow DNS Servers to resolve queries where the client sends a name to the DNS Server to request the IP address of the requested host,”[11] and DNS zone transfers, which are a “type of DNS transaction that’s one of the many mechanisms available for administrators to replicate DNS databases across DNS servers.”[12]

Next was DNS records. DNS records are “instructions that live in authoritative DNS servers and provide information about domains” such as the “IP addresses associated with domains and how to handle requests for domains.”[13] Another record mentioned was a CNAME (Canonical Name) record, which is a “type of DNS record that maps an alias name to a true or canonical domain name” and are typically used to “map a subdomain to the domain hosting that subdomain’s content.”[14]

Lastly Internet Information Services (IIS) were part of the lab. These are “flexible, general-purpose web servers that serve requested HTML pages or files.” IIS web servers “accept requests from remote client computers and return appropriate responses, allowing web servers to share and deliver information across corporate intranets (LANs), and the Internet (WANs).”[15]

The multiple parts of Domain Name Systems are helpful for businesses as they keep an organisations information inside the private network of that company. The different network resources allow information to be shared, help with emails, mapping, and managing the resources.

5.2.    Key Configurations and Commands

This week’s lab exercise was about an introduction to DNS. During the lab exercise, the following configurations and commands were learnt:

- ipconfig /flushdns: Used to clear any IP addresses or DNS records from the VM’s cache.

- Installing DNS: Log into swin22DC1  on sWin22SVR1 go to Server Manager  Manage  Add Roles and Features  install the DNS Server role with all default settings  close the wizard  Server Manager  Tools  DNS  DNS Manager console.

- Create a primary Forward Lookup zone: DNS Manager  expand SWIN22SVR1  right-click Forward Lookup Zones  New Zone  Next  Zone Type  Primary zone  Next  enter the Zone name burne.edu  Next  accept the default Zone file name and location  Next  do not allow dynamic updates  Next  Finish.

- Create a network resource in file server: On sWin22SVR3 launch File Explorer  This PC  create a folder in Local Disk (C:)  name the folder Data  create a text document called TopSecret.txt, add some text to it and save it  right-click the Data folder  Properties  Sharing  Advanced Sharing  Share this folder  limit the number of users to 10  Permissions  configure the Read permissions for Everyone  OK twice  Close.

- Create a network resource in web server: On sWin22SVR3 go to Server Manager  Add Roles and Features  Select server roles  add the Web Server (IIS) role  close the wizard  click Start and type Notepad  right-click and run as administrator  type something in Notepad  save the text file as index.htm in the C:\inetpub\wwwroot folder.

- Create a DNS record: On sWin22SVR1 go to DNS Management  Forward Lookup Zones  burne.edu primary zone  right-click the burne.edu zone  New Host  enter www in the Name field  make sure the FQDN is www.burne.edu  enter an IP address in IP address field  Add Host button  OK.

- Create a CNAME DNS record: On sWin22SVR1 right-click the burne.edu zone  New Alias (CNAME)  enter FileSvr in the Alias name field  make sure the FQDN is FileSvr.burne.edu  click the Browse button to set the target host  double-click sWin22SVR1 in Records  double-click Forward Lookup Zones  double-click the burne.edu zone  select the www Host (A) record  OK twice  DNS Management.

- Testing DNS: On sWin10PC203 log on as Admin  Ethernet adapter settings  add sWin22SVR1’s IP address as the preferred DNS server  Microsoft Edge  type www.burne.edu  File Explorer  type \\FileSvr.burne.edu  enter Administrator as the username and Pa55w.rd as the password  double-click the Data folder.

- Zone transfers: On sWin22DC1 load DNS Management  right-click Forward Lookup Zones  New Zone  Zone name burne.edu  Next  Master DNS server  enter sWin22SVR1’s IP address  Next  Finish  double-click the new burne.edu zone  on sWin22SVR1 go to DNS management  right-click burne.edu  Properties  Zone Transfers  Name Servers  Add sWin22DC1.swin.local  Resolve  OK  Zone Transfers  Allow zone transfers only to servers listed on the name servers tab  OK.

5.3.    Further Study

The lecture 05 revision quiz had questions related to Windows Server networks, DNS queries and DNS records and zones. This revision quiz was pretty simple, but I still got some questions wrong. One question included matching DNS records to the correspond description. I don’t feel like this is extremely important since you don’t need to know the description to configure it, but if these are going to be important to remember, then I should focus on remembering them. Another topic I would need to study are DNS queries and what queries occur between servers. Aside from that, I don't feel like I need to study many of the topics mentioned in the revision quiz.

- Week 6

6.1.    Key Concepts

Week six was about Active Directory Domain Services, which are “hierarchical structures that store information about objects on the network.”[16] These ADDS objects include domain accounts like domain user accounts, domain computer accounts, and domain group accounts.

Domain user accounts  “enable the service to take full advantage of the security features.” The account or “any group that the account is a member of” gets granted “local and network access.”[17] Domain computer accounts “are members of the Domain Computers Active Directory group and are often added to Active Directory groups for purposes of group policy management.”[18] Domain group accounts are “a collection of domains in the Manager that share the same configuration and ‘behave’ in the same way.”[19]

This lab also taught us how to assign permissions, and why assigning permissions is important, as they allow administrators to determine what type of access a user can have, and what the user has access to. Active Directory Domian Services help businesses manage the different domain elements of their network, which is important for optimization.

6.2.    Key Configurations and Commands

This week’s lab exercise was about configuring a Windows Server 2022 domain. During the lab exercise, the following configurations and commands were learnt:

Create a domain: On sWin22SVR3 go to Server Manager  Manage  Add Roles and Features  add the role of Active Directory Domain Services  accept default settings  Server Manager  click yellow alert symbol  Promote this server to be a domain controller  add a new forest  call the domain sWin.local  Domain Functional Level and Forest Functional Level  Windows Server 2022 level  accept default options for installing  restart the computer and log in again as sWin\administrator after installing.

Create a resource: Create two folders at C:\ called sWinData and Home  create text documents for sWinData  right-click the Home folder  Properties  Sharing  Advanced Sharing  Share this folder  Permissions  Full Control.

Join a domain: On sWin10PC203 log on as Administrator  ensure that the DNS address is configured with sWin22SVR3’s IP address  Start screen  System  click System  System window  Change Settings  Member of Domain field  type sWin.Local  OK  enter the username and password of an account that has permissions to create computer accounts in the domain  reboot when prompted.

Create a user account: On sWin22SVR3 run Active Directory Users and Computers from the administrative tools  right-click Users  New  User  enter Jill St John for the name and Jill for the user log on  Next  enter Pa55w.rd for the password  confirm password  clear the checkbox next to User must change password at next log on  go back to sWin10PC203 and log on as sWin\Jill  on sWin22SVR3 repeat the steps to create an account called Jack.

Create a computer account with DSA.msc: On sWin22SVR3 go to Active Directory Users and Computers  right-click the Computers container  New  Computer  type sWin10PC201 in Computer name field  Change  type Jill  Check Names  OK.

Set user account properties: On sWin22SVR3 go to Active Directory Users and Computers  Users container  right-click Jack  Properties  configure specified settings.

Set computer account properties: On sWin22SVR3 go to Active Directory Users and Computers  Computers container  right-click sWin10PC203  Properties  General tab  enter sWin10PC203 Jill’s virtual machine in the Description field  Location  enter 477B-101.

Create an organisational unit: On sWin22SVR3 go to Active Directory Users and Computers  right-click the sWin.Local domain root  New  Organizational Unit  name the OU AccountDept  remove the tick from Protect container from accidental deletion.

Create resource groups: On sWin22SVR3 right-click the AccountDept OU  New  Group  Domain local  devise a naming scheme that meets conventions  create two DL groups for the sWinData folder  give one DL Read Only access  give one DL Read Write access.

Assign NTFS permissions: On sWin22SVR3 right-click the folder C:\sWinData  Properties  Security  Edit  Add  Enter the object names field type the first three letters of the resource group names  click Check Names to list the matching groups  click Read Write version of your resource group  OK  add the Read Only version of your resource group  OK.

Remove inherited permissions: On sWin22SVR3 click the Security tab of the sWinData folder Properties  Advanced  Disable inheritance  convert inherited permissions into explicit permissions  Permission entries  click on every entry for the User group  Remove  OK twice.

Assign share permissions: Sharing  Advanced Sharing  tick Share this folder  Permissions  assign the Everyone group Full Control  OK three times.

Create user accounts: Active Directory Users and Computers  right-click the AccountsDept OU  New  Group  ensure the group scope is Global  name the group G_Acc_Mgrs  OK  right-click the new group  Properties  Members tab  Add Jill as a member of the G_Acc_Mgrs group  repeat and name the group G_Acc_Pay and make Jack a member of this group.

Nest groups: Add G_Acc_Mgrs group as a member of the DL_SWinData_RW group  right-click the G_Acc_Pay group  Properties  click the Member of tab  add the DL_SWinData_RO group as a member of group  OK.

6.3.    Further Study

The lecture 06 revision quiz was about ADDS permissions, forests, domains, OUs, and Role Based Access Controls. I didn’t really do well on any of the questions since I find questions related to these topics difficult and confusing because they seem to be similar but different at the same time. I know I need to focus on these topics more so I can understand them and answer questions involving them.

- Week 7

7.1.    Key Concepts

Week seven was about Active Directory Groups and Permissions and how it’s used to control access to network resources through Role Based Access Control (RBAC). By creating and organising accounts, access can be restricted to specified users. Moreso, week 7 focused on managing and organising these accounts using forests, nesting, organisational units, and more.

Part of the RBAC are child domains, which are domains “in domain trees whose Domain Name System name is a subdomain of a parent domain.”[20] Aside from that, the rest of the lab focused on the organisation aspect. A forest, “the topmost logical container in an Active Directory configuration that contains domains, users, computers, and group policies,”[21] was one of the organisational features.

Others included organisational units (OUs), “containers within a Microsoft Windows Active Directory domain that hold users, groups and computers,”[23] and nesting, which is “the process of putting one group inside another group.” By nesting groups, the groups will “inherit the permissions and privileges of the group they are put under.”[24] Lastly is domain controllers. Domain Controllers are “servers that manage networks and identity security”[22] protecting resources in a domain, and each domain controller will have options.

Active Directory Groups and Permissions are another way businesses can organise a network. Organisation makes a setup efficient as you know where everything is kept, meaning you won’t spend a lot of time looking for certain information. A business knowing where everything is will make administration easier.

7.2.    Key Configurations and Commands

This week’s lab exercise was about increasing object security using AD groups. During the lab exercise, the following configurations and commands were learnt:

Create a child domain: On sWin22SVR3 verify that the DNS server address is properly configured by ensuring you can successfully ping sWin.local  add the Active Directory Domain Services role  Server Manager  click the alert  Promote this server to be a domain controller  Deployment Configuration wizard  deployment operation  add a new domain to an existing forest  Select  provide the Enterprise Administrator's credentials  enter sWin.local\Administrator as the username and Pa55w.rd as the password  fill out the rest of the page with the specified information  Next  Domain Controller Options page of the wizard  enter Pa55w.rd as the DSRM password  Next  DNS Options wizard page  tick Create DNS delegation  ensure the Credentials for delegation creation is sWin.local\Administrator  Next four times  Prerequisites Check page  Install.

repadmin /syncall: Use if you receive an error message of failing to promote sWin22VR3. Used to “synchronize a specified domain controller with all of its replication partners.”[25]

Create network resources: On sWin22DC1 create the folders C:\Policy_Docs, C:\CustAccountData, and C:\Home  share these folders with the share permissions Everyone = Full Control  create sample files in the Policy_Docs and CustAccountData folders  remove inherited permissions for the Users group on the Policy_Docs and CustAccountData folder.

Create an OU: On sWin22DC1 go to Active Directory Users and Computers  right-click sWin.local  New  Organizational Unit  name the OU Finance.

Create a sWin.local group: Right-click the Finance OU  New  Group  name the group G_FinanceHwn  ensure that the Group scope Global is selected and that the Group type is Security.

Create a sWin.local user account and template: On sWin22DC1 launch Active Directory Users and Computers  right-click the OU Finance  New  User  enter _UsrTmpAccPay as both the first and user log on name  Next  tick User must change password at next log on and Account is disabled  Next  Finish  right-click _UsrTmpAccPay  Properties  Profile  Home folder to Connect H: to \\sWin22DC1\Home\%username%  Member Of tab  add G_AccPay  Account tab  set logon Hours to Monday to Friday  log On to sWin10CL101  account expires = the last day of the current year  OK.

Copy a template: On sWin22DC1 right-click _UsrTmpAccPay  Copy   enter the First name Doug, the Last name Pirahna and the User logon name dpirahna  Next  enter the Password Pa55w.rd  remove the ticks User must change password and Account is disabled.

Verify a child domain: Type Pa55w.rd to log on as WTNR\Administrator on sWin22SVR3  Server Manager  Tools  Active Directory Domains and Trusts  expand sWin.local  verify that Wtnr.sWin.local has been added as a child domain.

Join a domain: On sWin22SVR3 load PowerShell  redircmp "ou=Finance,dc=Wtnr,dc=sWin,dc=local"  enter  on sWin10PC203 ping the new child domain Wtnr.sWin.local  join sWin10PC203 to the Wtnr.sWin.local domain  use the credentials Wtnr\Administrator and the password Pa55w.rd  reboot  verify that its Computer account is appearing in the Finance OU in the Wantirna domain.

Nest within the Hawthorn domain: On sWin22DC1 go to Active Directory Users and Computers  Finance OU  nest the groups according to the Group Strategy.

Nest groups between domains: On sWin22DC1 go to Active Directory Users and Computers  double-click U_FinanceDept  Members tab  Add  Select Users  Contacts  click Locations  expand the sWin.local domain  select Wtnr.sWin.local  OK  Enter the object names  type G_  click Check Names  select G_FinanceWtn  OK.

7.3.    Further Study

The lecture 07 revision quiz continued touching on ADDS. The topics in this quiz had to do with tools and variables related to accounts, what statements are true about groups, resolving problems, access-based enumeration, commands, special identities, and combining permissions. The questions relating to special identities, commands, and combining permissions were alright for me, as I didn’t have trouble with those. The rest of the questions I did struggle with. These answers for the questions relating to access-based enumeration and statements applying to groups have similar wording, which is why I think these are difficult for me to understand. I’ll be focusing on the topics I got wrong, especially the topics that have a lot of answers with similar wording.

- Week 8

8.1.    Key Concepts

Week eight focused on centralising the configuration and security of computers connected to an Active Directory network through Group Policy Objects, which are “collections of Group Policy settings that define what systems will look like and how they will behave for defined groups of users.”[26]

Within GPOs are local policies that “provide administrators with a hierarchical tree structure for configuring Group Policy settings.”[27] Part of the local policies is GPO linking which “enables administrators to implement Group Policy settings for a broad or narrow portion of the organization.”[28]

Filtering GPOs was mentioned as filtering “allows you to limit the scope of a GPO based on certain criteria.”[29] Another GPO object was GPO modelling which “gives system administrators a chance to see how group policy would be applied to users and computers under different conditions without having to actually make those changes.”[30] The last thing the lab looked at was deploying printers with GPOs as “it helps to automate the user setup process.”[31]

Similar to the previous labs, Group Policy Objects are another way to manage and organise a network by centralising and managing configurations, operating systems, and settings. As well as helping a business keep their network organised, GPOs will also keep their network and information secure.

8.2.    Key Configurations and Commands

This week’s lab exercise was about group policy objects. During the lab exercise, the following configurations and commands were learnt:

Harden a desktop: On sWin10PC201 log on as Administrator  Start screen  type gpedit.msc  select gpedit.msc to load the Local Group Policy  Computer Configuration  expand Administrative Templates  expand Windows Components  double-click Autoplay Policies  double-click Turn off Autoplay policy  click Enabled option  enter the comment “Policy set to prevent malware being copied by autorun scripts”  set the Turn of Autoplay on: setting to All drives  OK  double-click Set the default behaviour for Autorun  click Enabled option  enter a comment  set the Default Autorun Behaviour option to Do not execute any autorun commands  OK.

Prohibit access to the control panel: On sWin10PC201 go to the Start screen  Control panel  select it to run  once verified close the Control panel  gpedit  User Configuration  expand Administrative templates  Control Panel  double-click Prohibit access to the Control Panel and PC settings  enter a comment  click Enabled option  OK  Start screen  Control Panel  observe whether it still appears in the Best match list.

Ctrl+Alt+Delete options: Verify that you have this option by pressing Ctrl+Alt+Del (Ctrl+Alt+End in a guest machine)  Cancel  Local Group Policy Editor console  User Configuration  Administrative Templates  System  Ctrl+Alt+Del options container  double-click Remove Lock Computer  Enable  enter a comment  OK  Ctrl+Alt+Del (Ctrl+Alt+End in a guest machine)  repeat the steps for the Remove Task Manager option.

Create an organisational unit structure: On sWin22DC1 go to Active Directory Users and Computers  create an OU called ICT  create two child OUs ICTSupport and ICTProcurement within the ICT OU  move one of the user accounts to the ICTSupport OU  move one of the users to the ICTProcurement OU  move any remaining user accounts and groups to the ICT OU.

Delegate control of an organisational unit: Right-click the ICTSupport OU  Delegate Control  start the wizard  add the user account that you copied into the ICTSupport child OU  delegate Reset user passwords and force password change and next logon  delegate Create, delete and manage groups  delegate Modify the membership of a group  finish the wizard.

Create a custom console: Launch MMC  add the Active Users and Computers snap-in  expand the sWin.local domain and get to the ICTSupport OU  right-click the ICTSupport OU  New Window from Here  Window menu  Console Root\ window  without closing the whole console, close this window  save the console as ICTSupportConsole.

Create group policy objects: On sWin10PC201 sign out and log on as a sWin\domain user whose user account is located in the ICTProcurement OU  verify that the Task Manager option does not appear when we press Ctrl+Alt+Del  on sWin22DC1 go to Server Manager  Tool  Group Policy Management  expand Domains  expand sWin.local until you see Group Policy Objects  right click Group Policy Objects  New  enter the name Enable Task Manager  right-click the new GPO  Edit  expand User Configuration  Policies  Administrative Templates  System  Ctrl+Alt+Del options  Remove Task Manager properties  Disabled option.

gpupdate /force: Used to refresh all of the settings that have changed since the last application of GPOs.

gpupdate /boot: Used to “cause a computer restart after the Group Policy settings are applied.”[32]

Link a GPO to a domain: On sWin22DC1 and sWin10PC201 type gpupdate  Group Policy Management  right-click the sWin.local domain  Link an Existing GPO  Enable Task Manager GPO  OK  run gpupdate /force on both sWin22DC1 and sWin10PC201  view your Ctrl+Alt+Del options on sWin10PC201  create a GPO called Remove Task Manager on sWin10PC201  link it to the ICTSupport OU  Remove Task Manager setting  Enabled  run gpupdate /force on both guest machines.

More GPO settings: On sWin22SVR1 log in as one of the user accounts created  run calc.exe  on sWin22DC1 in GPMC create and link a GPO called RestrictRunningOfCalc to the sWin.Local domain  edit the GPO  User Configuration  Policies  Administrative Templates  System  configure the Don’t run specified Windows applications policy setting to prevent users from running calc.exe.

Link multiple GPOs: GPMC  link the Remove Task Manager GPO and the Enable Task Manager GPO to the ICTProcurement OU  on sWin10PC201 log in as the user account located in the ICTProcurement OU  ICTProcurement OU in GPMC  select the Enable Task Manager GPO  click on the up arrow.

GPMC filter GPOs: GPMC  delete the link between the ICTProcurement OU and the Enable Task Manager GPO  link the Remove Task Manager GPO to the ICT OU  Group Policy Objects  click the Remove Task Manager GPO  Security Filtering settings  it currently has the Authenticated Users group configured  Remove  Add  G_ICTProcurement.

Filter with GPO security: GPMC  right-click the Restrict Running of Calc GPO  Edit  at the top left of the GPO right click on the GPO name  Properties  Security  Domain Admins group  Permissions  scroll down until you can see the permission Apply group policy  tick the Deny column for the Apply group policy permission  Remove Task Manager GPO  Security  Authenticated Users  ensure that your GPO has Allow Read permissions assigned  restrict the scope of the Remove Task Manager GPO by making sure that the Allow Apply group policy permission is blank.

Model GPOs: Bottom left-hand pane in GPMC  right-click Group Policy Modelling  start the wizard  select the defaults for the Domain Controller Selection  User and Computer Selection  enter the ICTProcurement user and the sWin10PC201 computer  tick Skip to the final page  Next  ignore any warnings about not trusting this page  Details tab to see what GPOs are being applied.

Create a new printer: GPMC  create a new GPO called IctPrinterDeploy  link it to the ICT OU  Server Manager  Tools  Printer Management console  expand Print Servers  expand sWin22DC1 (local)  right-click Printers  Add Printer  the Network Printer Installation Wizard will start  Printer Installation  Add a new printer using an existing port  port LPT1  Next  Printer Driver  Install a new driver  Next  Printer Installation  Brother Colour Leg Type1 printer  Next  Printer Name and Sharing Settings  name the printer ICT_Printer  share it as ICT_Printer  Next  Printer Found  Next  click Finish when the driver and printer have successfully installed.

Set printer permissions: Right-click the printer ICT_Printer  Properties  Security  give the group G_ICTSupport the Manage this printer and Manage documents permissions the same way you would when allocating NTFS permission  OK until you’re back at Print Management.

Deploy a printer using a GPO: Print Management  right-click ICT_Printer  Deploy with Group Policy  Browse button next to the field GPO name  find the IctPrinterDeploy GPO  OK  select the Checkbox this GPO applies to per user  Add  verify the UNC for the printer appears in the Printer Name column  OK  log on to sWin22SVR1 as one of the users created above (execute a gpupdate /force if you’re already logged in)  Settings  Devices  Printers & Scanners  verify that the printer has been deployed.

8.3.    Further Study

The lecture 08 revision quiz was about GPOs. The questions I was alright at were what changes are saved and what GPOs are linked, GPO filtering and settings, and administrative templates. The questions I didn’t do well at were the cumulated permissions, and the questions I got wrong were effective permissions, GPO linking, and applying GPOs. Even though I didn’t do too well on this quiz, I thinks it’s one of the quizzes I’ve done better on since I watched the lectures before class. Regardless of the questions I get right or wrong, I would need to study a lot before the exam, so I would go over everything to solidify my knowledge.

- Week 9

9.1.    Key Concepts

Week nine touched on managing security in active directory to improve the baseline security of a Windows Server 2022 Domain. Settings mentioned in this lab were default domain controllers, restricted groups, auditing, and more.

The default domain controllers policy is a “default container that is automatically created when a server is promoted to a domain controller, and is linked to the domain controller's OU and manages security settings for all domain controllers in a domain.”[33] The next settings mentioned was about restricting groups to “allow the administrator to configure local groups on client computer,”[34] and auditing, “a way of tracking and reviewing activities on your server, which you can then examine for potential security threats.”[35] More settings explored included the Encrypted File System (EFS), which “provides cryptographic protection of individual files on NTFS file system volumes using a public-key system.”[36]

The later part of the lab focused more on settings with GPOs, like WSUS settings with GPOs, and GPOs with firewalls. WSUS “provides features that you can use to manage and distribute updates through a management console,”[37] and firewall which is “a stateful host firewall within the that helps secure your device by creating rules to manage inbound and outbound network connections.”[38]

Lastly was Best Practices Analyzer. The Best Practice Analyzer or Security Compliance Manager is a “utility for analysing and comparing sets of GPOs which let you treat a set of GPOs as a single unit, making it easy to determine whether particular settings are duplicated across the GPOs or are set to conflicting values.”[39]

All of these settings and parts of servers help businesses track and manage their settings and users, ensuring no unwanted changes or activities are happening. Similarly, the security settings allow administrators to keep the server as secure as possible, as well as tailor the settings to the needs of the server or company.

9.2.    Key Configurations and Commands

This week’s lab exercise was about managing security in active directory. During the lab exercise, the following configurations and commands were learnt:

gpresult: Used to “displays the Resultant Set of Policy (RSoP) information for a remote user and computer.”[40]

Security settings: On sWin22DC1 go to Group Policy Management  edit Default Domain Controllers Policy  configure specified settings in Computer Configuration, Policies, Windows Settings, Security Settings  create a GPO called Lab9SecSettings  link it to the ICT OU.

Test the settings: Launch sWin10CL101 and sign in as swin\administrator  sign out  log on as swin\IPuser  Start  type cmd  right-click Command Prompt  run as administrator  enter the credentials sWin\Administrator with the default password  change to the IPuser's home folder by typing cd c:\users\IPuser in command prompt  type gpresult /user IPuser /h CL101GpoSettings.htm in command prompt  Enter  using File Explorer to browse to C:\Users\IPuser  double-click CL101GpoSettings1.htm  scroll down until you find the Applied GPOs section  Active Directory Users and Computers  move sWin10CL101 from the Computers container to the ICT OU  reboot sWin10CL101  log in as IPuser  run gpresult /h CL101GpoSettings2.htm from a command line.

Restrict a group: On sWin22DC1 create and link a GPO called RestrictAdminGroup at the sWin.local domain level  edit the Computer Configuration, Policies, Windows Settings, Security Settings, Restricted Groups policy  right-click Restricted Groups  Add Group  Administrators  Properties  Members of this group  add the user account ISuser  OK until you are back at the Group Policy Management Editor  Active Directory Users and Computers  Builtin container  right-click Administrators  Members tab  add the user account from the G ICTProcurement Group  OK  add the user account from the G ICTSupport Group  OK  gpupdate /force  Members tab of the Administrators group.

Auditing: On sWin22DC1 go to Default Domain Policy in GPMC  Computer Configuration, Policies, Windows Settings, Security Settings, Local Policies, Audit Policy  configure the specified settings  go to C:\Data  TopSecret.txt  advanced security settings Auditing  Add  Select a principal  enter Authenticated Users  select All in Type field  OK twice  reboot sWin10CL101 and log in as ISuser  go to \\sWin22DC1\Data  open TopSecret file, make some changes and save it  launch Event Viewer  expand Windows Logs  Security log  Find  enter in the user's logon name ISuser.

Use the encrypted file system: Ensure TopSecret.txt is closed on sWin10CL101  right-click the file  Properties  General tab  Advanced  Encrypt contents to secure data  OK  Properties  Apply.

WSUS settings with GPOs: On sWin22DC1 go to Default Domain Policy  Computer Configuration, Policies, Administrative Templates, Windows Components, Windows Update  make the specified changes.

GPO with firewall: Create a GPO called Firewall-AllowPing on sWin22DC1 and link it to the domain  Computer Configuration, Policies, Windows Settings, Security Settings, Windows Defender Firewall with Advanced Security  expand until you see Inbound Rules  right-click Inbound Rules  New Rule  configure the specified settings.

Best practices analyzer: Go to Server Manager on sWin22DC1  All Servers  select SWIN22DC1   scroll down to Best Practices Analyzer  click the drop-down arrow next to TASKS  Start BPA Scan  Start Scan button.

9.3.    Further Study

The lecture 09 revision quiz was about auditing, defence-in-depth, and other security topics mentioned in lecture 09. I didn’t do well with any of the questions, but I did watch the lecture and take notes. I didn’t focus on this lecture as much as I did on the last one, so I didn’t remember anything. Before the exams, I’ll take time to study all of the security topics, and everything else, and focus on the information I think will be important.

- Week 10

10.1.    Key Concepts

This week was about going over the overview of the skills exam, specifically part a, subnetting, and part b, group strategy design, which were the concepts addressed in this lab.

Subnetting design involves “dividing the allocated address space into smaller allocations for each subnetwork within the organization.”[41] Group strategy design relates to Active Directory Domain Services as part of ADDS are Active Directory Groups. To create an active Directory Group, we use IDGLA. IGDLA stands for Identities, Global groups, Domain Local groups, and Access.

Identities are “user and computer accounts that represent roles in an organization” that are nested inside of the Global groups. Global groups are “members of Domain Local groups which represent management rules.” These are nested inside of the Domain Local groups which “grant members access to resources based on the policies of the local domain.” Domain Local groups are nested inside of the Access which is “assigned to a resource that trickles down the hierarchy until it finds the identities that require it.”[42]

10.2.    Key Configurations and Commands

This week’s lab exercise was about looking at an overview of the skills exam, specifically the subnetting design and group strategy design. During the lab exercise, no configurations or commands were learnt.

10.3.    Further Study

The lecture 10 revision quiz was about quality management, disaster recovery, and monitoring networks. This is a quiz I did pretty well in, but there are still things I need to cover, like counters and thresholds. Questions related to these have been in a quiz before, and I still got it wrong, so this is obviously something I need to study further. Aside from counters and thresholds, I’m confident I have enough knowledge about the other topics.

- Week 11

11.1.    Key Concepts

This week was about doing the first practice skills exam, which involved subnetting design, group strategy design, configuring VMs according to the subnet and group strategy design, and extended configuration exercises. All of the content in the practice exam related to stuff we’ve been learning throughout the semester; therefore the lab didn’t address any new concepts or pieces of technology.

11.2.    Key Configurations and Commands

This week’s lab exercise was the first practice skills exam. During the lab exercise, the following configurations and commands were learnt:

icacls <drive_letter>:\<folder_name>: Used to “display or modify Access Control Lists (ACLs) for files and folders.”[43]

11.3.    Further Study

The lecture 11 revision quiz touched on IPv6 and cloud and virtualisation technologies. I got a few marks on this quiz, but not enough to say I did well. There were questions related to Saas, Iaas, Paas, and while I know what the acronym means, I don’t know the definitions. The IPv6 topics were about identifying addresses, compression them, or converting them. Aside from compression an IPv6 address, I’m not good at identifying the type of address or the corresponding IPv4 address. I’ll need to revise all of the topics in lecture 11 to get a good understanding and enough knowledge for the exams.

12.   Week 12

12.1.    Key Concepts

This week was about doing the second practice skills exam, which had different subnet and group strategy design, but still involved configuring VMs according to the subnet and group strategy design, and extended configuration exercises. All of the content in the practice exam related to stuff we’ve been learning throughout the semester; therefore the lab didn’t address any new concepts or pieces of technology

We also did a practice theory assessment that consisted of 36 questions, with some being essay style questions. These were all related to everything we’ve been learning throughout the semester, and even though this is the case, I still need to review everything, or the things I’m not confident at. There were no binary, broadcast address or subnet ID related questions when I was hoping there would be.

12.2.    Key Configurations and Commands

This week’s lab exercise was the second practice skills assessment. During the lab exercise, the no new configurations or commands were learnt.

12.3.    Further Study

This week there were no revision quizzes as it was the last week. We did another practice skills exam, and as mentioned in week 10, the skills exam involved subnetting and group strategy design. These are things that I will need to study further. The subnetting scenario given was:

sWin an international IT solutions company, is launching 19 new branches in a new country where they currently have no existing branches. The sWin CIO has asked that each subnet should host at least 75 devices. sWin has obtained a network address of 128.92.0.0/18. This address needs to be subnetted further. The internal subnet mask will be /23. The company’s subnet plan allows for:

- Router Pool: 2 router interfaces for each subnet – the first 2 IP addresses from each subnet.

- Managed Network Device Pool: The first 10 IP addresses that follow the Router Pool are reserved  for switches and other network devices

- Server Pool: The first 10 IP addresses following the Managed Network Device Pool are reserved for servers.

- Workstation Static Pool: The first 5 IP addresses following the Server Pool are reserved for Workstations that require static IP addresses.

- Workstation DHCP Pool: The remaining IP addresses are reserved for automatic IP configuration

You have been asked to use subnet 5 for your device addressing. To calculate the subnets, first convert the subnet masks from slash notation to decimal:

| /18 = 255.255.192.0 | /23 = 255.255.254.0 |
| --- | --- |

Both masks are in the third octet, so we’ll be working in the third octet.

Minus the allocated mask by 256, so 256 - 254 = 2. This means we’ll be counting up by 2 in the third octet. Start at subnet 0 (the original address), which is 128.92.0.0.

| 128.92.0.0 | Subnet 0 |
| --- | --- |
| 128.92.2.0 | Subnet 1 |
| 128.92.4.0 | Subnet 2 |
| 128.92.6.0 | Subnet 3 |
| 128.92.8.0 | Subnet 4 |
| 128.92.10.0 | Subnet 5 |

We have been asked to use subnet 5 for your device addressing, so our address becomes 128.92.10.0. Next look at the criteria and allocate the addresses accordingly.

The next part was the group strategy design. The scenario given for the group design was:

sWin Ltd wants to react more quickly to market demands. It is determined that the Marketing data must be available to all Marketing personnel. Also, sWin Ltd executives must be able to view the data. SWin Ltd wants to create the group structure for the entire Marketing division, which includes the Research and Advertising departments. The Advertising department will need read/write access to the Advertising data and read only access to the Research data. Similarly, the Research division will need read/write access to the Research data and read only access to the Advertising data.

Figure 6: Group strategy design.

References

[1] 	“Virtual Machine,” VMware, https://www.vmware.com/au/topics/glossary/content/virtual-machine.html#:~:text=A%20Virtual%20Machine%20(VM)%20is,a%20physical%20%E2%80%9Chost%E2%80%9D%20machine (accessed Aug. 19, 2023).

[2] 	NAVIKO Team, “How to Install and Configure Hyper-V Manager,” Nakivo, https://www.nakivo.com/blog/install-configure-hyper-v-manager/#anc_0 (accessed Aug. 19, 2023).

[3] 	“Azure Lab Services - Cloud Virtual Machines,” Microsoft Azure: Cloud Computing Services, https://azure.microsoft.com/en-us/products/lab-services (accessed Aug. 19, 2023).

[4] 	Libby, “What is IPv4 Address and its Role in the Network?,” Study CCNA, https://study-ccna.com/what-is-ipv4-address/ (accessed Aug. 22, 2023).

[5] 	D. Walker and R. Speed, “What is a default gateway?,” ITPro, https://www.itpro.com/network-internet/30327/what-is-a-default-gateway (accessed Aug. 22, 2023).

[6] 	ButterCashier, “MAC address,” Wikipedia, https://en.wikipedia.org/wiki/MAC_address (accessed Aug. 22, 2023).

[7] 	J. Gerend, K. Downie, E. Ross, I. Prittie, and J. McIllece, “Dynamic Host Configuration Protocol (DHCP),” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/networking/technologies/dhcp/dhcp-top (accessed Aug. 23, 2023).

[8] 	“DHCP Scopes,” SolarWinds, https://documentation.solarwinds.com/en/success_center/ipam/content/ipam-create-dhcp-scopes.htm#:~:text=A%20scope%20is%20a%20consecutive,IP%20addresses%20to%20DHCP%20clients (accessed Aug. 23, 2023).

[9] 	C. Sanders, “DHCP Reservations and Exclusions,” TechGenix, https://techgenix.com/DHCPReservationsandExclusions/ (accessed Aug. 23, 2023).

[10] 	J. Gerend et al., “ipconfig,” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/administration/windows-commands/ipconfig (accessed Aug. 22, 2023).

[11] 	D. Hannifin, J. Alpern, and N. J. Alpern, “Chapter 3: Windows Server 2008 R2 networking,” in Microsoft Windows Server 2008 R2 Administrator’s Reference, Rockland, MA: Syngress, 2010, pp. 106–106.

[12] 	S. Leeds, “DNS zone transfer,” Wikipedia, https://en.wikipedia.org/wiki/DNS_zone_transfer (accessed Sep. 10, 2023).

[13] 	DNS records, https://www.cloudflare.com/learning/dns/dns-records/ (accessed Sep. 10, 2023).

[14] 	“About CNAME records,” Google Workspace Admin Help, https://support.google.com/a/answer/112037?hl=en#zippy=%2Cset-up-cname-records-now (accessed Sep. 10, 2023).

[15] 	L. Rosencrance and S. J. Bigelow, “Internet Information Services (IIS),” SearchWindowsServer, https://www.techtarget.com/searchwindowsserver/definition/IIS (accessed Sep. 10, 2023).

[16] 	I. Foulds et al., “Active Directory Domain Services Overview,” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/identity/ad-ds/get-started/virtual-dc/active-directory-domain-services-overview (accessed Sep. 21, 2023).

[17] 	A. Ashcraft et al., “Using a domain user account as a service logon account,” Microsoft Learn, https://learn.microsoft.com/en-us/windows/win32/ad/domain-user-accounts (accessed Sep. 21, 2023).

[18] 	S. Metcalf, “Sneaky Active Directory Persistence #16: Computer Accounts &amp; Domain Controller Silver Tickets,” Active Directory Security, https://adsecurity.org/?p=2753#:~:text=Computer%20accounts%20are%20members%20of,computer%20accounts%20to%20AD%20groups (accessed Sep. 21, 2023).

[19] 	C. support, “What are Domain Groups and how should I use them?,” Cookiebot CMP, https://support.cookiebot.com/hc/en-us/articles/360003725253-What-are-Domain-Groups-and-how-should-I-use-them-#:~:text=A%20Domain%20Group%20is%20%E2%80%93%20as,behave'%20in%20the%20same%20way (accessed Sep. 21, 2023).

[20] 	Editor, “Child Domain,” NETWORK ENCYCLOPEDIA, https://networkencyclopedia.com/child-domain/ (accessed Sep. 21, 2023).

[21] 	M. Buckbee, “What is an Active Directory Forest?,” Varonis, https://www.varonis.com/blog/active-directory-forest (accessed Sep. 21, 2023).

[22] 	D. Worthington, “What is a Domain Controller?,” JumpCloud, https://jumpcloud.com/blog/what-is-a-domain-controller (accessed Sep. 21, 2023).

[23] 	A. Zola, “What is Organizational Unit (OU)?,” SearchWindowsServer, https://www.techtarget.com/searchwindowsserver/definition/organizational-unit-OU (accessed Sep. 21, 2023).

[24] 	B. Narayanan, “Active Directory Nested Groups Explained,” Windows Active Directory, https://www.windows-active-directory.com/nesting-groups-in-active-directory.html (accessed Sep. 21, 2023).

[25] 	“Repadmin /syncall,” Microsoft Learn, https://learn.microsoft.com/en-us/previous-versions/windows/it-pro/windows-server-2012-r2-and-2012/cc835086(v=ws.11) (accessed Sep. 12, 2023).

[26] 	L. Rosencrance and A. Kucharik, “What is group policy object (GPO) and why is it important?,” SearchWindowsServer, https://www.techtarget.com/searchwindowsserver/definition/Group-Policy-Object#:~:text=Microsoft’s%20Group%20Policy%20Object%20(GPO,Policy%20Management%20Console%20(GPMC) (accessed Sep. 21, 2023).

[27] 	“Local Group Policy Editor,” Microsoft Learn, https://learn.microsoft.com/en-us/previous-versions/windows/it-pro/windows-server-2012-r2-and-2012/dn265982(v=ws.11) (accessed Sep. 21, 2023).

[28] 	“Linking GPOs to Active Directory Containers,” Microsoft Learn, https://learn.microsoft.com/en-us/previous-versions/windows/desktop/policy/linking-gpos-to-active-directory-containers (accessed Sep. 21, 2023).

[29] 	Windows System Administration, “What are the advantages and disadvantages of using GPO filtering and delegation?,” GPO Filtering and Delegation: Pros, Cons, and Tips, https://www.linkedin.com/advice/1/what-advantages-disadvantages-using-3c#:~:text=GPO%20filtering%20can%20be%20done,Policy%20Preferences%20(GPP)%20editor (accessed Sep. 21, 2023).

[30] 	E. O. Neale, “Chapter 20: Group Policy,” in Microsoft Small Business Server 2003 Unleashed, Indianapolis, IN: Sams, 2005, p. 482.

[31] 	R. Allen, “Deploy Printers With Group Policy,” Active Directory Pro, https://activedirectorypro.com/deploy-printers-with-group-policy/ (accessed Sep. 21, 2023).

[32] 	J. Gerend et al., “gpupdate,” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/administration/windows-commands/gpupdate (accessed Sep. 21, 2023).

[33] 	“1.1.9 Group Policy Management,” Microsoft Learn, https://learn.microsoft.com/en-us/openspecs/windows_protocols/ms-gpod/566e983e-3b72-4b2d-9063-a00ebc9514fd (accessed Oct. 8, 2023).

[34] 	“Group Policy Restricted Groups,” ITfreetraining, https://itfreetraining.com/lesson/gp-restricted-groups/#:~:text=Restricted%20Groups%20allows%20the%20administrator,all%20clients%20on%20your%20desktop (accessed Oct. 8, 2023).

[35] 	S. Contributor, “Server auditing best practices: Windows, SQL, and File Server Auditing,” DNSstuff, https://www.dnsstuff.com/server-auditing-windows-sql-file-servers-auditing (accessed Oct. 10, 2023).

[36] 	A. Ashcraft et al., “File Encryption,” Microsoft Learn, https://learn.microsoft.com/en-us/windows/win32/fileio/file-encryption (accessed Oct. 10, 2023).

[37] 	J. Gerend et al., “Windows Server Update Services (WSUS),” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/administration/windows-server-update-services/get-started/windows-server-update-services-wsus (accessed Oct. 10, 2023).

[38] 	D. Fuchs, “How To Manage Windows Firewall with GPOs,” Blumira, https://www.blumira.com/windows-firewall-with-gpos/ (accessed Oct. 10, 2023).

[39] 	V. Pamnani, “Microsoft Security Compliance Toolkit - How to use,” Microsoft Learn, https://learn.microsoft.com/en-us/windows/security/operating-system-security/device-management/windows-security-configuration-framework/security-compliance-toolkit-10 (accessed Oct. 10, 2023).

[40] 	J. Gerend et al., “gpresult,” Microsoft Learn, https://learn.microsoft.com/en-us/windows-server/administration/windows-commands/gpresult (accessed Oct. 10, 2023).

[41] 	T. Slattery, “How to calculate a subnet mask from hosts and subnets,” Networking, https://www.techtarget.com/searchnetworking/tip/IP-addressing-and-subnetting-Calculate-a-subnet-mask-using-the-hosts-formula (accessed Oct. 20, 2023).

[42] 	S. Fortner, “IGDLA and you: A Fledgling’s Guide to Nesting,” Medium, https://medium.com/tech-jobs-academy/igdla-and-you-a-fledgling-s-guide-to-nesting-e09b748e3a60 (accessed Oct. 20, 2023).

[43] 	“Cacls.exe,” Cacls - Modify Access Control List - Windows CMD - SS64.com, https://ss64.com/nt/cacls.html (accessed Oct. 20, 2023).